Dear Leslie,

- Concerning the new Fréjus laboratory:

In grey, this is the present Fréjus road tunnel, and in black a possible design of a Safety tunnel. What is new concerns the Safety tunnel: in March 06 the French-Italian Safety Committee has endorsed a design with a diameter of 8.20m; then 21st of April 06 the French-Italian intergovernmental Commission has accepted a useful diameter of 8,00m. Since 2001, the diameter of this Safety tunnel has been a sort of “saga” and, officially this 8,00m size is justified just to use large firemen trucks (ie. not to increase the road traffic)...

Needless to say that now the two governments should find money to proceed to the excavation of this urgently needed Safety tunnel. One needs 4 years to build it afterwards. This is a very good news, as 8,00m is largely a good size to speed up the MEMPHYS cavities excavation (air ventilation and crushed rock evacuation): 4-5 years.

- Concerning the Detector cavity:

Engineering pre-study has been undertaken by the SETEC and STONE Companies which has built the present road tunnel. The large set of rock quality measurements made along the tunnel has been used to determine the best location and the cavity dimensions. The main results are:

- the best site (rock quality) is found in the middle of the mountain, at a depth of 4800 m.w.e : a really good chance !

- of the two considered shapes : “tunnel” and “shaft”, the “shaft shape” is strongly preferred

- Cylindrical shafts are feasible up to: a diameter F = 65 m  and a full height  h = 80 m (≈ 250 000 m3). The main ingredient here is the width of the plastic deformed zone which must not exceed 10m (half the main anchor length). Beyond that maximal length the feasibility is harder with non-standard anchors.

- with “egg shape” or “intermediate shape” the volume of the shafts could be still increased (although not yet extensively studied)

- The estimated cost of each cavity excavation with the concrete and anchors included is ≈ 80 M€.

- Concerning the Detector design

The detector is adjusted to the result of the engineering pre-study described in sec. 2. One shaft may host a 65m x 65m cylinder filled with water which correspond to 216kT. The PMTs surface is 2m from the walls and 2m more is required a a guard to define the Fiducial volume of 145kT. The inner volume is viewed by about 81,000 12”-PMTs which constitute 30% coverage (but equivalent to 40% coverage by 20”-PMTs as 12”-PMTs are supposed to have better Q.E and C.E). The Veto Outer volume is viewed by 8”-PMTs. The target cost of a complete photodetection unit (PMT + electronics + H.V) is 1k€, so each shaft may cost ~ 80k€. IN2P3-CNRS Laboratories (LAL-LAPP-IPNO) and Photonis Co. are in close contact to undertake a R&D program along this line as soon as funds are given.

- Schedule

The tentative schedule already shown in some circumstances is the following and more indicative

Year       2005                      2010                        2015                       2020

Safety tunnel

Excavation

Lab cavity

Excavation

P.S

Study

detector

PM R&D

PMT production

Det.preparation

Installation

Outside lab.

Non-acc.physics

P-decay, SN

Superbeam

Construction

Superbeam

betabeam

Beta beam

Construction

decision for cavity digging decision for SPL  construction decision for EURISOL site

- Strategy

The most recent news concerns the EL SUD (European Large Scale Underground Detector) initiative which groups the GLACIER, LENA and MEMPHYS communities in order to propose and undertake common R&D in cavity excavation study, cavity equipment (radio-protection), Safety, fluid manipulation and treatment, photodetectors, electronics, DAQ as well as Physics potential studies.  We are currently writing a “Scientific Case” document and at NNN06 it may happen that a talk will be given to announce this initiative. It is foreseen to request Funds to the forthcoming European Funding Plan (FP7).