- To set up the environment

  - Connect on the opera machine

  - setenv /exp/opera

  - source config.csh

  - cd <work-area>

  - cd OSArea/MEMPHYS/<latest-version>/cmt

    - later the path <work>/ OSArea/MEMPHYS/<latest-version> will be referred to MEMPHYSROOT (which by the way is a shell environment varaible)

  - source setup.csh

- To run interactively

  - Cd MEMPHYSROT and then “svn update” to be up to date wrt the repository

  - To compile/link: (cd ../cmt ; gmake)

  - The event file is in  MEMPHYSROOT/data: for instance the file memphys_water.kin3 has been generated by NUANCE (http://nuint.ps.uci.edu/nuance/default.htm). The Geant4 simulation read this file through the class “PrimaryGeneratorAction”.  See Appendix A for some information concerning NUANCE.

  - cd MEMPHYSROOT/run

    - be sure that the symbolic link “inputEventfile” points to the right input data file

  - to lunch the program

    - ../Linux-i686/MEMPHYS_session.exe –stdout

    - it  will  send some debug message and then after some initialization phase (~45sec) :

*****************************************************

Geant4 version Name: geant4-08-00    (16-December-2005)

Copyright : Geant4 Collaboration

Reference : NIM A 506 (2003), 250-303

WWW : http://cern.ch/geant4

*****************************************************

ConstructMaterial: Dump Water properties

(JEC) Set12inchPMTGeometry DUMP:

WC Total Z (m) 65 WC Total R (m) 32.5 Cap Z  (m) 0.44 Veto Total (m) 2 Barrel Z (m) 60.56 Barrel PMT R (m) 30.5

Cov (%)= 30 Surf Cath (cm2) 615 Cell Surf (cm2) 2050

Cell Approx Length (m) 0.452769 Real Cell Z (m) 0.455338 rPhi at PMTsurf (m) 0.453043

Nz 133 Nphi 423 N Barrel PMTs = 56259

WCCapEdgeLimit (m) 30.67

Secondary interaction model set to BINARY

(JEC) ConstructWC Nber of Cap Cells = 14108

Rock : RockLength = 28000

WC Position = 0

RockLength     = 28 m

WCLength       = 65 m

expHallLength = 139.5 m

DetectorConstruction::GetWCGeom: -3050  -100000

DetectorConstruction::GetWCGeom: -3050  3050

Thank you for using G4BinaryCascade. /exp/opera/kits/Geant/geant4.8.0/data/G4NDL3.7

Thank you for using G4BinaryCascade.

PhysicsList::SetCuts:CutLength : 1 mm

Input vector file set to inputEventfile

/run/verbose 2

/event/verbose 0

/tracking/verbose 0

/MEMPHYS/tracking/drawing/optical true

/hits/verbose 0

    - The event display starts

      - On the Geant4 menu select Run…

      - Set 10 events then Apply

      - New verbose debug will appear and will stops at the end of each event by “Play with the event”

      - The Viewer_1 panel will be filled with the optical photon if  /MEMPHYS/tracking/drawing/optical true is set in the MEMPHYSROOT/scripts/Geant4/init.g4m configuration file. To play with the event the most efficient way to learn is to have a demo by G. Barrand.

- To run in Batch

  - Do as for interactive procedure

  - Launch the program ../Linux-i686/MEMPHYS_batch.exe

  - In the MEMPHYSROOT/application/MEMPHYS_batch.src is set the number of events to process.

  - The output of this batch is Root file which is MEMPHYS.root

  - To extract some basic information there is 3 macros in the MEMPHYSROOT/run directory

    - MEMPHYS_read_event.C: read the MC event

    - MEMPHYS_hit.C: read the hit structure information

    - MEMPHYS_digit.C: read the digit structure information

    - MEMPHYS_geom.C: dump the geometrical position of the PMTs

    - All are running as: root –l <macro>.C and are skeleton to produce plots and so on.

  - There is the possibility also to output a geometrical file (text or ROOT file) in the Detector construction

Appendix A : NUANCE

An example event is shown below:

$ begin
$ nuance 3
$ vertex 811.5 -1030.4 -1681.7 5.86347E+04
$ track 14 1000.0000 1.00000 0.00000 0.00000 -1
$ track 2212 935.9840 -0.03219 -0.24434 0.96915 -1
$ info 2 949000 0.0000E+00
$ track 13 295.6312 0.15021 0.98821 -0.02950 -2
$ track 211 427.5731 0.66875 -0.66698 -0.32849 -2
$ track 2212 1212.7798 0.88658 -0.07320 0.45675 -2
$ track 13 295.6312 0.15021 0.98821 -0.02950 0
$ track 2212 1212.7798 0.88658 -0.07320 0.45675 0
$ track 111 382.1095 -0.73647 -0.35958 -0.57299 0
$ end

Each line begins with a text tag, indicating the information it contains.

|  | begin: Marks the start of the event. |
| --- | --- |
|  | nuance: This line indicates the reaction mode for the event. |
|  | vertex: The first three numbers are the x, y and z coordinate, in cm. The fourth number is the time, in simulation units (nominally seconds). If you are using a beam, it will be in the units you specified for the integrated luminosity. |
|  | track: The track lines contain most of the information. The last number on the line is either -1 (initial state particle), -2 (final state particle before interactions) or 0 (the final state particle after interactions). Normally your detector simulation should only track particles with a status code of 0. The first number on the line is the particle code, using Particle Data Group/Lund conventions. An exception is that for coherent reactions, an ad hoc particle code is assigned for the target nucleus, since the PDG system does not encompass nuclei. For nuclei, the code is Z*1000+A, so Oxygen is particle code 8016. The next number is the total energy of the particle. By default, this is in MeV. Starting with version 2.001 of the program, you can give an EGEV card to switch all output to GeV energy units. Note that for bound nucleon targets, the total energy will be less than the rest mass since the target is bound. Finally, the third, fourth and fifth numbers are the x, y and z direction cosines. |
|  | info: The first two numbers are obsolete and/or irrelevant and are retained only for compatibility with Super-K software. The third number is the neutrino flux for this energy and direction. The units are the same as whatever the input flux is. This can be useful if you later want to re-weight events (after they have passed through detector simulation and reconstruction). |
|  | end: marks the end of the event |
|  | Stop: (Not shown above) marks the end of the file. |

The particle codes used by NUANCE for some common particles are: -11 (e+), 11 (e-), -13 (mu+), 13 (mu-), 2212 (p), 2112 (n), 111 (pi0), 211 (pi+), -211 (pi-), 12 (nu_e), 14 (nu_mu), 22 (gamma).

The reactions known to the program are described by a reaction code.  The following is an overview of what the reaction codes mean:

| 1 | Charged current quasi-elastic |
| --- | --- |
| 2 | Neutral current quasi-elastic |
| 3-16 | Single-pion resonant production |
| 17-90 | Additional resonant channels with other final states |
| 91 | Charged current deep-inelastic |
| 92 | Neutral current deep-inelastic |
| 93-94 | Not used |
| 95 | Cabibbo-suppressed quasi-elastic scattering |
| 96 | Neutral current coherent/diffractive pion production |
| 97 | Charged current coherent/diffractive pion production |
| 98 | Elastic scattering from electrons |
| 99 | Inverse muon decay (electron target) |