J.Marteau

June 14th , 2004

Goal : this note describes the data to be recorded in the DAQ local database and the information needed to be duplicated in the OPERA central DB

DAQ reminder : the OPERA DAQ architecture makes use of Ethernet at the earliest stage of the readout chain. A daughter-board with a local processor is plugged on the digital mother-board of each sub-detector (Target Tracker, RPC & XPC tracker, drift tubes Precision Tracker, RPC Veto). It allows a direct connection of the digital boards to the so-called DAQ processes (T.T. DAQ 1 & 2, RPC DAQ and P.T. DAQ) which apply the 1st levels filters and event timestamps, perform the event building for each sub-detector and make insertions in the DAQ local DB of the filtered event sequences.

All the DAQ processes are inter-connected and connected to the DAQ Manager process. The DAQ Manager also receives the information of the GPS time (through a dedicated PCI decoder board plugged in the server) and of the local clock. This clock, consisting of a fast signal (20MHz) and a Pulse Per Second  (PPS), is distributed to all the nodes in the experiment to synchronize local clocks. The DAQ Manager acts also as an “absolute time server”.

The detector DAQ network is a standard Ethernet network with separate sub-networks for each sub-detector (T.T. 1 & 2, RPC, P.T.), for the DAQ Manager and for administration.

The DAQ servers and the DAQ sub-networks will be mounted in racks on the top platform of the experiment. The Manager server will be in the Control area in Hall C.

The local DAQ DB server could be in the same area or in the Terminal room in Hall B.

Slow Control  : the Slow Control parameters originate from local S.C. systems such as :

- RPC/P.T./magnets temperature + current + voltage sensors located at the level of each sub-detectors and readout with dedicated PC driven systems either on the top of the detector or in the Control area

- PM’s high voltage modules and temperature sensors managed by each T.T. digital board (data are available at the level of the DAQ processes)

- Racks and crates controller which record various parameters (status, current consumption, overload…) on a PC in the Control area through any serial link

- Gas system control located in the TIR gallery close to the Emulsion Processing facility

- BAM slow control parameters (BAM tunnel)

- BMS slow control parameters (Hall C &&&)

- Emulsion processing parameters (TIR gallery)

- Storage area parameters (Hall B)

- Environmental parameters (temperature & humidity probes located in Hall C)

- a.o.b.

Monitoring : the monitoring information consists in :

- trigger rates in each RPC chambers

- calibration parameters (time & charge measurements) in P.T. drift tubes

- calibration & online parameters (ADC chain, scintillator & PM gain, trigger rate) in each T.T. modules

- a.o.b.

Beam information : the assumed SPS supercycle lasts 34.8s. Its division among the various users (LHC filling, LHC setup, CNGS and Fixed Target experiments) will vary with time. During the CNGS physics run it is assumed that each supercycle will contain 3 CNGS cycles (36s), 1 FT cycle (112s) and 1 cycle for parallel Machine Development (14.8s).

The information concerning the beam should consist of  :

- a pre-beam warning (~4s before particles are delivered)

- a post-beam status released once the particles have been send

- a log from the monitoring stations of the beam

The beam related information should transit through the network (which should be guaranteed as efficient as possible between CERN and Gran Sasso). In principle, the detector can be put in “non-acquisition” mode (for calibration, slow control & monitoring, data transfer…) for a few seconds after the post-beam message is received. It turns back to the “acquisition” mode until the next post-beam message arrives.

From the detector side, we are requested to send :

- a warning message if the detector is not ready for data taking to avoid unnecessary irradiation of materials

- a daily log of the counting rate in the experiment (no beam profile or spectrum is requested)

All these information should be stored in a local DB (at least to cross-check the coherence of the event reconstructed timestamps and the timestamps of the pre- and post-beam messages).

Data format and singles rate :

Target Tracker. The data format is of the form :

TIME_CYCLE(16) + (TIME(32) + (CHANNEL(8) / ADC(16))*(NADC transmitted))*Nevents .

Assuming a singles rate of 50Hz per channel and 1 ADC in average, we get :

- 180kbits/s per PM (max. theo. 100Mbits/s)

- 2.9 Mbits/s per TT plane

- 46 Mbits/s per TT DAQ server (max. theo. 1Gbits/s)

- 184 Mbits/s for OPERA (max. theo. 1Gbits/s)

RPC Tracker. The data format is of the form : START(32)/DATA(2*32)/STOP(32). Assuming a rate of 7kHz singles (extrapolated from measurements in Gran Sasso on small scale prototypes) we get :

- 448 kbits/s per plane (max. theo. 100Mbits/s)

- 10 Mbits/s per RPC DAQ server (max. theo. 1Gbits/s)

- 20 Mbits/s for OPERA (max. theo. 1Gbits/s)

Precision Tracker. The data format is of the form : HEADER(40)/DATA(16)/CHANNEL(8). Assuming a rate of 14kHz (OR of 2 RPC planes) and an average of 3 noisy channels giving a signal in coincidence with the trigger we get :

- 1.6 Mbits/s per TDC board (max. theo. 100Mbits/s)

- 76 Mbits/s per PT DAQ server (max. theo. 1Gbits/s)

- 152 Mbits/s for OPERA

# Data rate and transfer to the DB

Target Tracker.

A rough estimate of the random coincidences (500Hz for a TT plane in the 100ns coincidence gate) + the radioactivity measured in Gran Sasso (0.3Hz) leads to : ~800 events/s/TT plane. These events contain 4 ADC in total (~225bits). In total it leads to 180kbits/s/TT plane and 12Mbits/s for OPERA. Most of these events should be filtered by the 1st level Trigger.

The 1st level of Trigger requires at least a X-Y coincidence with more than 5+5 p.e. or 2 X-Y coincidences in 2 consecutive planes with more than 1+1 p.e. each time. The number of random + radioactivity events above this threshold (and therefore stored in the DB) will be very low.

For the nominal neutrino event rate, we assume (an optimistic) 60 events per day (0.7mHz). In average each of them gives 69 hit strips (~8kbits) for a CC interaction and 40 hit strips (~4.5kbits) for a NC interaction. Assuming a 2:1 ratio we have 60 hit strips in average per neutrino interaction. The total amount of neutrino data will be of the order of  400kbits/day.

On top of these neutrino interactions from the beam, we will collect events coming from neutrino interactions in the surrounding rock, muons (1/m2/h), atmospheric neutrinos…

Monitoring data. On top of these data we have a continuous local storage of the singles by means of 64 histograms (one per channel). In total we have 64*256*16bits ~ 250kbits transmitted every 1000 acquisition cycle (~1000s) and stored in the DB. For the 1000 PM’s, it leads to around 250kbits/s of monitoring data to be stored in the DB.

Slow control. The slow control parameters (HV value and temperature of the boards) are transmitted to the DAQ in a routine mode (typically every acquisition cycle). It leads to an overall data rate of 100kbits/s for the detector.

In total the data rate is (in Mbits/s) :

12 (low energy coincidences) + 0.25 (monitoring) + 0.1 (slow control) + 0.000002 (neutrino events) + ??? (atmospheric, cosmics…) < 15 Mbits/s.

Note that most of the low energy coincidences will not pass the 1st level filters.

RPC Tracker.

Monitoring data. The monitoring is performed through a continuous storage of singles for each channel (544 channels per plane). Assuming an average rate of 7kHz per plane (overestimated for the inner planes shielded by the iron slabs), we have ~7Hz per channel. Monitoring histograms, transmitted every 100s are ~400kbits size.

The level of data filtering at the level the RPC DAQ server is not yet defined.

Precision Tracker.

The level of data filtering at the level the PT DAQ server is not yet defined.

Figure 1 : the OPERA underground activities.

Figure 2 : the OPERA DAQ network