12-02-2003

UPDATING OF SHORT SPECIFICATION OF A HORN PULSER SYSTEM FOR A NEUTRINO-FACTORY

S.Gilardoni, G.Gräwer, G.Maire, J-M.Maugain, S.Rangod, F.Völker

1. Introduction

The main design parameters retained for a prototype NUFACT horn are the following:

Maximum current 			: 	300 kA

Pulse repetition rate: 			50 Hz

Waist diameter			: 	80 mm

Free waist aperture diameter	:	56 mm

Outer diameter			: 	400 mm

Life time				:	6 weeks or 2 x 108 pulses for the prototype

( six months or 8 x 108 pulses for a final horn)

Pulse length			:	 100 μs

r.m.s current			: 	15 kA

Voltage on horn			:	~ 4000 V

Capacitor charging voltage	:	 7000 V

Skin depth				: 	1.25 mm

Joule losses				: 	40 kW

Very high radiation resistance for insulator materials

With respect to existing horns, the challenge consists mainly in providing technical solutions adapted to :

- more acute heating problems due to the high repetition rate.

- longer life time ( two orders of magnitude higher number of pulses).

# 2.  Prototype horn constructed at CERN and ready for tests

A prototype horn ( see Fig.1, Fig.2, Fig.3 ) has been produced at CERN to assess feasibility. The inner conductor of the horn (inner current conducting skin) is cooled with a water spray and in addition by convection with an annular water-cooling film flowing along and between the inner conductor and an additional external Aluminium skin. This horn is thus denominated « double skin » horn.

Inner and outer conductors and flanges are made of Al alloy (AA 6082-T6).

Fig.1 : Horn constructional view

Fig. 2  Prototype horn on test

Fig. 3  Electrical circuit ( in a possible final version)

# 3. Electrical parameter list

This horn is intended to be operated in final according to following table of electrical parameters :

|  | Units | Prototype Horn |
| --- | --- | --- |
| Distance of capacitor and switching unit location from horn | m |  10 |
| Peak current in horn | kA | 300 |
| Pulse repetition rate | Hz | 50 |
| Inductance of horn (max.) | µH | 0.3 |
| Additional inductance | µH | 0.4 |
| Total inductance (max.) | µH | 0.7 |
| Resistance of horn at 100 0C (with skin effect) | µ | 180 |
| Additional resistance | µ | 320 |
| Total resistance (max.) | µ | 500 |
| Total capacitance of capacitor bank | µF | ~ 1400 |
| Pulse duration (half period) | µs |  100 |
| Skin depth | mm | 1.25 |
| Charging voltage | V |  7000 |
| Energy stored in capacitor bank | kJ | ~ 35 |
| Stored energy recovery rate | % | 86 |
| Voltage on element | V | < 4000 |
| r.m.s. current in horn | kA | 15 |
| Mean power dissipation in horn by current (*) | kW | 40 |
| Water flow needed in horn for w  7 °C * | l/min | 82 |
| Number of pulses per month of operation |  | 1.3 x 10 8 |

(*) The power dissipation due to beam absorption has to be added to this figure.

# 4. Electrical lines

A detailed study of the connecting lines between the power supply and the horn has not yet been done. The line impedance which adds to the horn impedance is nevertheless of prime importance. For the electrical calculations one has assumed a total additional inductance of  0.4  µH and a total additional resistance of  320µ including electrical lines, connections , internal capacitor bank and discharge components.

# 5. Tests done at CERN in 2002 with available resources

## Test at 1Hz, 30 kA, 100 s

The horn has been powered at the repetition rate of  1 Hz  using a 1kV charging power supply and two parallel energy storage and switching branches. Due to high parasitic inductance in the discharge circuits, and in order to keep the specified pulse duration, two third of the full (1400 F) capacitor bank has been used in the two branches, reaching a  peak current of 30kA in the horn.

Pulse Repetition Frequency (PRF) 	1 Hz

Imax 					30 kA

C 					1000 F

Pulse duration 			100  s

Charging voltage 			1000 V.

Stored energy in capacitors:		500 J

Mechanical vibration modes have been measured in the horn (fundamental  frequency 195 Hz).

# 6. Tests foreseen at CERN in 2003 using existing resources

## Test at 1Hz, 100 kA, 150  s pulse duration

The aim is to pulse the horn at a repetition rate of  1 Hz by charging  a  2800 F capacitor bank up to  2000 V.

The  bank  will be discharged through the same thyristor branches (with reduced parasitic inductance) so as to reach a current peak of up to 70-100 kA with a pulse duration of         150 s.

#### Test at 0.25 Hz, 250 kA to 300kA, 2 to 5 ms pulse duration

The aim is to deliver pulses of up to 300 kA into the horn. To this end the horn needs to be connected to an existing pulse transformer (turns ratio 10 or 32). The construction of a short, low cost, quadruple-stripline for connection to the horn is foreseen.

Two CNGS capacitor banks will be discharged, through two thyristor branches of the CNGS test circuit, into the primary of the transformer, so as to reach a peak current of 300 kApeak in the horn [i.e. 300 kA/(10 or 32) in the primary of the pulse transformer], with a pulse duration between 2 ms and 5 ms.

The precise working point acceptable to all circuit components has not yet been fixed.

This test will also provide first results concerning the efficiency of the cooling system.

# 7. Final horn fatigue tests to be envisaged with resources yet to be found

## Test at 50 Hz, 300kA, 100 s

### A fast charging 7 kV power supply ( i.e. a prototype unit for~10 A/7000V and a final one     for 50 to maximum 100 A depending on the energy recovery rate and of the line length ) capable of recharging within  20ms a 1400 µF capacitor bank is needed. This capacitor bank will be split into a number of branch cubicles (e.g. 10), each  one  including its own thyristor switch-assembly designed for operation at 7 kV.  A low inductance and low resistance stripline and horn connection arrangement must be studied and constructed.

Resources for this entire test program are missing at the time being.

Reference: 	CERN NUFACT Note 80  -  21st May 2001

Study of a horn with integrated target for a Neutrino factory

J.M. Maugain, S. Rangod, F. Voelker