# The Multi-MegaWatt Pion Production Target and Pion Collector

1. Introduction

High intensity neutrino beams (superbeams neutrino factories and muon colliders) require a high power target as a pion source and an efficient pion collection system. Developments in high power targets are also needed in several different scientific and technological areas - linear collider positron sources [], radioactive beam facilities [], pulsed spallation neutron sources [], accelerator production of tritium [] and accelerator transmutation of radioactive waste []. Synergies already exist between some of these groups. However, pion collection is specific to neutrino beams. The intimate relation of target and collector mean that that they must be designed as a unit.

2. Target Design

The main problems of the target design are: heat dissipation, radiation damage and, in addition, with pulsed beams, thermal stress, shock and fatigue. The high radiation can bring problems of heating, activation and radiation damage to surrounding components.

The key issue is not total power dissipation but power density and, for pulsed beams, energy density per pulse. A high power dissipated in a large volume produces a small temperature rise and the heat is easily dissipated; similarly the radiation damage is low, although it can build up with time. The transmutation and tritium production facilities [] have the highest levels of beam powers - in the tens to hundreds of MW – but with the very large targets the energy density is comparatively low and hence the heating and radiation damage problems are eased.

The current design of neutrino target [] will be bombarded by a 4 MW proton beam of 3-30 GeV, pulsed at 50 Hz with a pulse structure which will consist of one or more 1 ns long micro-pulses within a 1 μs macro-pulse. The heavy metal targets are 1-2 cm in diameter and 20-30 cm long. The length is dictated by the desire for an acceptable pion beam emittance and the diameter is restricted by self absorption of the pions.  The target will dissipate ~1 MW of the beam power the remainder being dissipated in a beam dump. The energy density is 300-1000 J cm-3 per pulse, which is one of the highest of currently proposed targets for any type of machine.

To reduce the effective energy density, the target can be moved through the beam. Rotating solid targets are in use [] and a contained flowing liquid metal targets is being made at PSI [].  The SNS [] is also committed to a contained flowing mercury target. However, the SNS has a pulsed proton beam, unlike the cw PSI beam, and the liquid metal suffers from shock effects. In particular, cavitation bubbles are formed and those near the surface erode the container wall. Currently, the expected lifetime is only a few weeks at 1 MW [].

Another solution is to form a free metal jet which will disintegrate in the beam pulse, and then reform between pulses. The jet has many advantages since heating, radiation damage and shock are not a problem. However safety issues, containment and thin beam windows could present problems.

To reduce the thermal shock in solids it is advantageous to have a small target so that the transit time for the shock is small compared to the pulse length. For pulses of 1 ns length this requires a size of a few mm. Using this principle, a target consisting of small beads has been suggested by Sievers [].

There is some freedom to choose the pulse configuration of the proton driver. If the macro-pulse is split up into several micro-pulses separated by at least the shock transit time then the energy density per pulse is effectively reduced by the number of micro-pulses in the macro-pulse. It appears feasible to have at least 10-20 micro-pulses thus reducing the energy density per pulse by an order of magnitude and reducing the effect of shock damage in the target accordingly.

2.1. The Granular Target

Peter Sievers [] suggested a target consisting of small solid tantalum spheres (~2 mm diameter), cooled by flowing gas or liquid (see Figure 1). This reduces the effect of thermal shock that is experienced by a target constructed from a single piece of solid material. To further reduce the energy density in the target it was proposed to sweep the beam across 4 targets on successive pulses and to collect the pions and recombine them into a single beam in a sophisticated arrangement of optics []. Currently, work on this target has ceased at CERN.

Figure 1. Schematic section through the granular target.

2.2. Solid target Studies in the USA

BNL are currently investigating materials (various nickel iron alloys) that have low thermal expansion to circumvent the problem of thermal shock. It has been found that under irradiation the materials loose the low expansion properties, although they can be recovered by modest heating. The search for better materials is continuing.

2.3. Solid Target Studies in the UK

The proposed target []  for the neutrino factory is a rotating tantalum toroid (see Figure 2) operating at a temperature of ~2000 K to dissipate 1 MW of power produced by the impact of a 4 MW proton beam. Alternatives to the toroid are being considered - such as firing individual target bars through the beam or passing a “chain” of bars through the beam.

rotating toroid

proton beam

solenoid magnet

toroid at 2300 K radiates heat to water-cooled surroundings

toroid magnetically levitated and driven by linear motors

Figure 2. Schematic diagram of the rotating solid tantalum toroid

The UK programme [3] of target developments is centred on the study of thermal shock and the long term fatigue effects. A high current electric pulse will be passed through a thin tantalum wire to provide a thermal shock by ohmic heating. The radial acceleration of the surface of the wire will be measured by a VISAR [4] and the motion of the surface will be modelled using a commercial computer code, LS-DYNA [5]. In this way the constitutive equations of the tantalum at high temperatures under shock conditions will be realised and the full scale target can be modelled.

Finally, some in-beam testing is planned to confirm the VISAR measurements with the wire test and the resultant modelling. Tests at FNAL on the pbar target have already shown no disastrous effects at an energy density of 38000 J cm-3 (two orders of magnitude higher than the neutrino factory) for 1100 pulses.

Tantalum has been chosen for the toroid because it is refractory and resistant to radiation damage []. However, the possibilities of carrying out studies of radiation damage at high temperature are currently being investigated.

2.4. Liquid Metal Jet Target Studies

Mercury has been chosen as the liquid for the jet although low melting point lead alloys are possible equally good alternatives. The jet is formed from a nozzle located at the entrance to the collection system. The jet is placed at a small angle (~100 mrad) to the beam to reduce pion self absorption in the jet.

Tests have been carried out at BNL [] to test the effect of an intense beam pulse on the jet and have confirmed that the jet does disintegrate with droplet velocities of 10-20 m s-1 (see Figures 4 and 5) The velocities increase with the energy deposited in the jet []. Reforming the jet between pulses was not a problem. Tests at Grenoble [] showed that the jet was made more stable in an intense solenoidal magnetic field of 20 T.

Currently an experiment, MERIT [], is being constructed to test the jet in both a magnetic field and with beam of the correct intensity (up to 28x1012 protons per pulse at 24 GeV) to simulate the neutrino factory conditions. This experiment is a collaboration between laboratories in the USA, Japan and Europe. It will be installed in the nToF11 beam line at CERN for operation in 2007.

Figure 4. BNL experiment. Schematic diagram of the mercury jet hit by a proton beam; no magnetic field.

Figure 5. Pictures of the mercury jet disintegrating when hit by the proton beam. Exposures of 25 μs at t = 0, 0.75, 2, 7, 18 ms after a 1 cm diameter mercury jet was struck by a pulse of 2x1012 protons at 24 GeV.

The solenoid magnet [] is being fabricated from copper and cooled with liquid nitrogen to ~80 K to reduce the resistance and ohmic losses. The magnet is pulsed with a slow rise over ~10 s to reduce inductance effects and has a flat few top of 1-2 s before slowly falling to zero. The cycle time between pulses is ~30 minutes to allow cooling to take place. The maximum field is calculated to be ~15 T.

The jet velocity will be >10 ms-1, ~1 cm in diameter. Currently studies of the nozzle design are being undertaken at Princeton. The jet and proton beam are off the solenoid axis; the jet is at an angle of 100 mrad and the proton beam at 67 mrad. The proton beam size will be ≤1.5 mm rms. The diagnostic of the beam-target interaction will be optical through four high speed cameras, while the secondary pion flux will be monitored by scintillator counters.

Only a few hundred pulses are required since, in principle, a single pulse will be able to show the success of the jet target concept. The goals of the experiment include studies of the jet entering the magnetic field, the jet dispersal by the beam in the magnetic field and the influence of the jet entry angle on jet performance at fields from 0 to 15 T. Also, the CERN PS beam profile allows one to study the effect of varying the charge intensity from 5x1012 to 28x1012 protons per spill and to study the effects of the jet dispersal on the beam in long pulses. If the mercury has not dispersed sufficiently it could absorb pions. It is not possible directly to test the jet at 50 Hz operation, but it is possible at reduced beam energy (14 GeV) to extract 2 spills separated by 20 ms. This will be an important part of the test to confirm the correct re-establishment of the jet on successive beam pulses.

The MERIT experiment is an important experiment which will hopefully show that reliable long-life high power targets can be built. There is still an issue of safety with radioactive mercury; the use of a low melting point metal alloy could improve this situation.

3. Pion Collection

Two options are discussed which relate to the neutrino facility: a superbeam would require a magnetic horn (see Figure 4), sign-selecting the pions to be focused in the decay channel, whereas a neutrino factory would favour a solenoid magnet focusing both sign pions at the same time.

From an initial design made at CERN on a horn prototype system (horn + reflector) [1] foreseen for an Neutrino Factory[], an optimization and a redesign has been made in a SuperBeam context [2] driven by the physics case of a long baseline experiment (130km) between CERN and Fréjus (MEMPHYS detector location).

Figure 4. Schematic section of the horn for pion collection.

The proton energy appears to be best suited between  3.5 and 4.5 GeV proton kinetic energy and the current is then required to be 300 kA in the horn and 600 kA in the reflector, for a conductor thickness of 3 mm to minimize the energy deposition by secondary protons in the neck (<30kW); a real challenge. Integration and test with a target would need a dedicated R&D effort in an experimental area designed to face the various safety aspects (chemistry of heavy metals, high radiation levels, high voltage, high current …). The power supply design is known as extrapolation of existing facilities, but requires special attention and cost estimate due to the high frequency envisaged, 50Hz. A dedicated facility with prototypes of the different electrical components and a load (eg. Horn prototype) is very suitable to test existing component provided by manufacturers.

Generally the solenoid collects more pions the stronger the magnetic field. A compromise between field and aperture has to be made. The US Design Study 2 [], indicates that a 20 T solenoidal magnet is feasible, with a reasonable life time. The solenoid has water-cooled copper coils in the centre, which also act as radiation shields to the superconducting coils.

The pion yield by 2-20 GeV protons is however still poorly known: Geant4 and Mars simulations give significantly different results (up to 50%) in the best phase space regions both for superbeams and neutrino factories. The HARP experiment is presently publishing its first results [3] and should help clarify this point soon.

4. Integration Issues

There is an awareness [] that many of the target problems are common to several communities. These issues are being discussed at a number of gatherings, such as the High Power Target Workshops series []; this is helping to bring the various communities together.

Groups working on neutrino factories around the world are already collaborating closely. A good example of this is the MERIT mercury jet experiment []; a collaboration between laboratories in the USA, Japan and Europe which reflects the worldwide effort to master the technological challenges towards a neutrino factory and a muon collider.

The absence of suitable in-beam test facilities is a severe problem. There has been a recent proposal [] from ORNL to try and set up an in-beam facility to allow target to be tested at high power.

Radiation damage to materials is another area requiring proton beams. Pulsed facilities are scarce but PSI [] is providing a good, although limited, service using cw proton beams.

Due to the absence of testing facilities and the dependence on other ongoing developments, in this R&D, the role of simulation is very important. A tuning of these simulations (deformations, radiation resistance, fatigue, power dissipation etc…) could be done using input provided by the previously described tests and facilities. This software development has to be done in parallel with all related hardware tasks.

What can we say about collector issues?

5. Recommendations

- Currently progress is seriously limited by resources, so more resources would be of great benefit.

- Nevertheless some progress is being made with targets, particularly mercury jets and studies at CERN, BNL and in the UK on solid materials. This should continue with more resources.

- The opportunity for more interactions with other communities needing high power targets must be continued and expanded.

- An in-beam test facility available to all should be pursued.

- Related simulations have to be done in parallel.

- Setup a dedicated area for the horn power supply at high frequency tests.

References

[1] 	A. Ball et al. CERN-NUFACT-Note-42

[2] 	J.E. Campagne CERN-NUFACT-Note-138

[3] 	HARP Coll. to appear in Nuclear Phys. B

[4] 	H.G.Kirk et al. Target studies with BNL E951 at the AGS, proc. 2001 Part. Accel. Conf. (Chicago, IL, June 2001), p. 1535.

[5]	J. Lettry et al., Thermal shocks and magnetohydrodynamics in high power mercury targets, J. Phys. G: Nucl. and Part. Phys. 29, 1621 (2003).