| Proposal full title | A Design Study for Large Apparati for Grand Unification and Neutrino Astrophysics |
| --- | --- |
| Proposal acronym | LAGUNA |

Table of contents

1.	Introduction	3

2.	Scientific goals and European added value of the new Infrastructure	4

3.	Scientific and Technological excellence	6

3.1.	Working packages and implementation plan	6

3.1.1.	WP 1: Underground infrastructures (site engineering)	6

3.1.2.	WP 2: Underground Tanks	8

3.1.3.	WP 3: Tank Instrumentation	9

3.1.4.	WP 4: Liquid procurement and handling systems	9

3.1.5.	WP 5: Safety & Environment	10

3.1.6.	WP 6: Underground science	11

3.1.7.	WP 7: Management and coordination	11

4.	Relevance to the Objectives of the scheme	11

4.1.	Justification of the proposed Design Study	11

4.2.	Exploring the feasibility of the infrastructure	12

5.	Quality of the Management	12

5.1.	Management and Competence of the Participants	12

5.2.	Justification of the Financing Requested	12

Table 1 – List of participants of the Design Study NOT YET ORDERED!!

| Participant number | Organisation | Short name | Short description (i.e. fields of excellence) and specific roles in the consortium |
| --- | --- | --- | --- |
| 1 | Centre National de la Recherche Scientifique / Institut National de Physique Nucléaire et de Physique des Particules (CNRS/IN2P3), France | IN2P3 | IN2P3 is a division of CNRS operating more than 20 laboratories around France charged with research in particle, astro-particle and nuclear physics. |
| 2 | ETH Zurich, Switzerland | ETHZ |  |
| 3 | DESY, Germany | DESY |  |
| 4 | University Granada, Spain | U-Granada |  |
| 5 | University Hamburg, Germany | U-Hamburg |  |
| 6 | TU Munchen, Germany | TUM |  |
| 7 | University Sheffield, UK | U-Sheffield |  |
|  | Laboratoire Souterain Modane | LSM |  |
|  | Center for Underground Physics at Pyhäsalmi mine | CUPP |  |
|  | Sierozsowice | SUNLAB |  |
|  | Laboratorio Subterraneo de Canfranc, Spain | LSC |  |
|  | Laboratori Nazionali del Gran Sasso, Italy ???? | LNGS??? | Will they join? |
| 9 | STONE s.p.a. | STONE | Feasibility studies of large caverns for Fréjus site |
| 10 | Technodyne Ltd | TECHNODYNE | Engineering bureau for the design of large warm and cryogenic tankers. |
| 11 | Photonis. France | PHOTONIS |  |
| 12 | Electron Tubes | ETL |  |
|  | Linde | LINDE |  |
| 13 | Institut Luft- Kältetechnik, Dresden, Germany | ILK |  |
|  | CAEN, Italy | CAEN |  |

List needs to be completed!

# Introduction

To be submitted to the specific programme of research, technological development and demonstration “CAPACITIES” / “Research infrastructure”

Period 2007-2013

To optimize the use and development of the best research infrastructures existing in Europe, and helping to create new research infrastructures of pan-European interest that provide essential services to the European scientific community.

To support the feasibility studies for new research infrastructures, which are of clear European dimension and interest.

They main concern the whole range of scientific and technological fields from social sciences to astronomy, and include for example libraries, databases, clean rooms, synchrotrons, accelerators or telescopes.

The infrastructures might be “singled-sited”, “distributed” or “virtual”.

The DS could develop into a construction of new infrastructures as reviewed by the European Strategy Forum on Research Infrastructures (ESFRI) roadmap, in a two-stage approach: (1) support for preparatory phase (finalization of the detailed construction plans, of the legal organization, of the management and multi-annual final planning; (2) support in the implementation phase, i.e. the actual construction, building on the technical, legal, administrative and final agreement achieved during the preparatory phase between all stakeholders. The ESFRI roadmap will be taken through periodic revisions and only projects that have sufficiently progresses in the preparatory phase and identified as such by ESFRI will proceed to phase 2.

The design study should address all key questions that will help to assess the technical and financial feasibility of the proposed new facility. The outcome of the work should be a conceptual design report. First prototypes can be included if appropriate.

Major upgrades of existing infrastructures may also be considered, when the end result is intended to be equivalent to, or be capable of replacing, a new infrastructure.

The design study should be carried out by at least three independent legal entities established in three different member states or associated states: in our case CH, DE, ESP, FR, FIN, POL, UK. International European Interest Organizations (IEIO) and the JRC may participate under the same conditions as legal entities from Member and Associated states. The DS is also open to participation to entities from third countries, which, in duly justified cases, may also receive a Community financial contribution.

Criteria 1: The extent to which

- the new facility is scientifically and technologically original and innovative;

- the proposed study is scientifically and technologically well structured.

Criteria 2: The extent to which

- the European significance and interest is demonstrated, in particular in terms of the needs of potential users or new opportunities being opened.

- the study addresses key questions which will help assess the technical and financial feasibility of the proposed new facility leading to a “conceptual design report”

Criteria 3: The extent to which

- the project management and the competence of each participant are appropriate for the intended study or work;

- there is a clear description of the planned work, a solid justification of the budget and the individual tasks are well coordinated in order to achieve optimal results.

# Scientific goals and European added value of the new Infrastructure

Fundamental research: A broad particle and astroparticle physics program

- Baryon number violation: Proton decay

- Astroparticle physics

  - Gravitational collapse: Supernova - 

  - Early alert for astronomers: Supernova -

  - Star formation in the early universe: Relic SN -

  - Solar thermonuclear fusion processes: Solar - 

  - Indirect dark matter searches: Muons, 

  - Neutrino properties: Supernova - , Atmospheric - , Long baseline - 

  - Geophysical models, Earth density profile: Atmospheric -  , Geo - 

Physics programme over 30 years

The adopted technologies

- Water Cerenkov imaging: …

- Liquid Argon imaging: The liquid Argon Time Projection Chamber (LAr TPC) is a powerful detector for uniform and high accuracy imaging of massive active volumes. It is based on the fact that in highly pure Argon, ionization tracks can be drifted over distances of the order of meters. Imaging is provided by position-segmented electrodes at the end of the drift path, continuously recording the signals induced. T0 is provided by the prompt scintillation light.

- Scintillator light imaging: …

Multidisciplinary research

Interest by EU organizations

ApPEC support:

We recommend that a new large European infrastructure is put forward, as a future international multi-purpose facility on the 105-106 ton scale  for improved studies of proton decay and of low-energy neutrinos from astrophysical origin.  The three detection techniques being studied for such large detectors in Europe, Water-Cherenkov, Liquid Scintillator and Liquid Argon, should be evaluated in the context of a common design study which should also address the underground infrastructure and the possibility of an eventual detection of future accelerator neutrino beams. This design study should take into account worldwide efforts and converge, on a time scale of 2010, to a common proposal.

Interest by non-EU organizations

CERN support

Japan

Russia

USA

# Scientific and Technological excellence

## Working packages and implementation plan

### WP 1: Underground infrastructures (site engineering)

Feasibility of large excavations, access, local conditions, site pre-selection

Feasibility studies (for all sites)

Including thorough rock sampling, rock simulations

Pre-plan for construction

Cost estimates

Site pre-selection

Work with Integrated Large Infrastructure for Astroparticle Science ILIAS-N2-WG1 and respective WG in ILIAS-next

Table 2 - Very preliminary sites vs experiments

|  |  | Mt Water Cerenkov | 50 kt Liquid Scintillator | 100 kt Liquid Argon |
| --- | --- | --- | --- | --- |
| Fréjus | Tunnel / hard rock | √√√ | √√ | √√ |
| Gran Sasso | Tunnel / soft rock | √ | √√ | √ |
| Canfranc | Tunnel | ? | ? | ? |
| Pyhäsalmi | Mine / hard rock | √ | √√√ | √√ |
| Boulby | Mine / salt (potash) | ? | ? | ? |
| Polkowice - Sieroszowice | Mine / salt & rock | √ | √√ | √√√ |
| Green fields | Own shaft / Hard rock | √ | √ | √√√ |

√√√ primary interest; √√ probably; √ unlikely; ? unknown

Results of the pre-feasibility study in the central region of the Fréjus tunnel

The site located in the Fréjus mountain in the Alps, which is crossed by a road-tunnel connecting France (Modane) to Italy (Bardonecchia), has a number of interesting characteristics making it a very good candidate for the installation of a Megaton-scale detector in Europe, aimed both to non-accelerator and accelerator based physics (proton decay and neutrinos from different kind of sources). Its great depth (4800 mwe), the good quality of the rock, the fact that its access is horizontal, its distance from CERN (130 km), the opportunity of the excavation of a second (“safety”) tunnel, the very easy and comfortable access by train (TGV), by car (highways) and by plane (Geneva, Torino and Lyon airports), the strong support from the local authorities both on the French and Italian side, represent the most important of these characteristics.

On the base of these arguments, the DSM (CEA) and IN2P3 (CNRS) Institutions decided to perform a feasibility study of a Large Underground Laboratory in the central region of the Fréjus tunnel, near the already existing, but much smaller, LSM Laboratory.

This preliminary study, partially (at the level of ≈ 30%) funded by the Region Rhône-Alpes,  has been performed by the SETEC (French) and STONE (Italian) companies and is now completed. These companies already made the study and managed the realisation of the Fréjus road tunnel and of the LSM Laboratory (both about 25 years old). A large number of precise and systematic measurements of the rock characteristics, performed at that time, have been used to make a pre-selection of the most favourable regions along the road tunnel and to constrain the simulations of the present pre-study for the Large Laboratory. Three regions have been pre-selected : the central region and two other regions at about 3km from each entrance of the tunnel. Two different shapes has been considered for the cavities to be excavated : the “tunnel shape” and the “shaft (well) shape” and the main purpose was to determine the maximum possible size for each of them, the most sensitive dimension being the width (the so-called “span”) of the cavities.

The very interesting results of this preliminary study can be summarized as follows :

1) the best site (rock quality) is found in the middle of the mountain, at a depth of 4800 mwe : a

really good chance !

2) of the two considered shapes : “tunnel” and “shaft”, the “shaft (= well) shape” is strongly

preferred

3) Cylindrical shafts are feasible up to a diameter  = 65 m  and a full height  h = 80 m

(≈ 250 000 m3)

4) with “egg shape” or “intermediate shape between cylinder and egg shapes” the volume of the

shafts could be still increased

5) The estimated cost is ≈ 80 M€ X Number of shafts

The figure below shows the envisaged configuration for this Large Laboratory, where up to five shafts, of about 250 000 m3 each, can be located between the road tunnel and the railway tunnel, in the central region of the Fréjus mountain.

As already said, the studied configuration considers the possibility to locate up to five shafts of bout 250 000 m3 each (and perhaps more!), but of course it is possible to excavate a smaller number of these shafts.

Two possible scenarios for Water Cerenkov detectors are, for example :

a) 3 shafts of 250 000 m3 each, with a fiducial mass of 450 Ktons (“UNO-like” scenario)

b) 4 shafts of 250 000 m3 each, with a fiducial mass of 600 Ktons

In both scenarios one additional shaft would be necessary for a Liquid Argon detector of about 100 ktons total mass.

The next step should be a Design Study for this Large Laboratory, performed in close connection with the Design Study of the detectors and considering the excavation of 3 to 5 “shafts” of about 250 000 m3 each, the associated equipments and the mechanics of the detector modules. Such a detailed study should include :

a) a precise “in situ” investigation of the rock quality parameters

b) the optimisation of the shafts shape

c) the optimisation (choice) of the type of local access

d) the definition of the required equipments : ventilation and air-conditioning, water purification “factory”, electric power supply, etc.

e) the definition of the best solution for the water containment and for the photo-detectors support.

### WP 2: Underground Tanks

Deliverables

To establish design of the underground tanks

Design, geometry, support structure, materials, insulation

To demonstrate the feasibility of underground assembly

Preliminary costing of underground construction

Milestones

Optimization of tank shapes (cylindrical vertical, horizontal tube) and modularity (including construction time)

Assess compatibility of tank with liquid (scintillator) in connection with WP4

Preliminary design of tank (including the possibility of lining)

Definition of construction strategy as a function of underground access method

Methods

Partnership and/or subcontracting to specialized industries (Technodyne, Rockplan, …)

Partnership with specialized civil and cryogenic engineering departments

Contact and/or partnership with company exploiting mine or road tunnel

Meetings among senior physicists, industry and specialized engineers

### WP 3: Tank Instrumentation

Deliverables

To establish design of large scale and low cost solutions for tank instrumentation, in particular:

- Design of light readout schemes for optimum light collection and cost in large volume liquid detectors for both room and cryogenic temperature

- Design of charge readout schemes for optimum charge collection and cost in large volume liquid detectors for liquid noble gas

- Large scale high voltage creation and distribution

- Large number of channels readout electronics and interface to data acquisition and storage

- Methods of calibration (amplitude and time), distribution of time synchronization for systems with large number of components

- Systems for distributed slow control

- Mechanical supporting structures

- Methods of accessing and if necessary replacing (part of) instrumentation

Milestones

Evaluation and optimization of best elements (best photo-detectors, best charge readout, …)

Solutions for large area devices

Assess necessity and if applicable mass production of light concentrators

Reach of industrial solutions for mass production

Proposals for distributed data acquisition, slow control, calibration and time distribution

Assessment of failure rates and aging effects

Methods

Laboratory tests of sensor prototypes

Cost reduction

Methods for mass production

New design of HV supply, signal decoupling, HV feed-through, …

New design of VHV for drift fields

Accelerated aging tests

Postdoc driven subtasks

### WP 4: Liquid procurement and handling systems

Deliverables

Definition of the way how to fill the detectors with large quantities of liquid argon, scintillator, and water and to reach and maintain the specifications (optical properties, purity and radiopurity) in large volumes

Milestones

Evaluation of procurements for large quantities of liquid argon, liquid scintillators, ultrapure water and ultrapure gases (like N2): Availability, costs, transport, and quality (transparency, light yield, radiopurity etc.).

Evaluation of the filling techniques of the detectors: Filling strategy and logistics (tanks, tubing etc.). Methods to avoid recontamination.

Evaluation of purification systems for argon, liquid scintillators, and ultrapure water:

Purification in order to achieve the goals for radioactive levels, optical properties.

Continuous purification (recirculation)

Evaluation of the compatibility with detector components, which are in contact with the liquids: Corrosion, radon emanation, long term stability of the optical and low radioactivity levels of the liquids (in connection with WP2).

Evaluation of the gas systems needed for maintenance and purification: Radon depleted N2 – supply for gas blanket and purification systems.

Evaluation of quality control methods: Development of in-situ measuring techniques for optical properties (link to calibration), light yield, and radiopurity.

Methods

Investigation of availability, costs, transport methods. Development and performance of test measurements for optical properties and radiopurity. Development and performance of accelerated aging tests for compatibility measurements. Development of in-situ calibration tools for optical properties. Development of in-situ measurement techniques for determent of the radiopurity levels.

Scintillators: Test measurements of the light yield, absorption and scattering lengths and the timing of various liquid scintillator samples. Test measurements of purification methods with solid columns (e.g. Si-gel, Al2O3), gases, liquids (e.g. water extraction). Development of systems for scintillator mixing with wavelength shifters. Development and tests of purification methods for wavelength shifters.

Water: Test measurements of water purification systems (e.g. reverse osmosis, deionization, distillation etc.). Determination of Ra, Rn, U, and Th – levels by different experimental methods. Tests of solubility and stability of Gd-compounds in water and the compatibility with detector components.

Liquid Argon: Development and testing of cryogenic systems for LAr handling and purification.

### WP 5: Safety & Environment

Deliverables

Definition of the additional infrastructure required for safe operation

Definition of the interface between the installation safety and the overall safety strategy of the host (tunnel or mine)

Milestones

Definition of the internal (liquid leaks) and external risks (stability of cavern, earthquakes, fire risks

Definition of risks of personel

Methods

Subcontracting studies (risk analysis, etc.) with safety experts

Coordination with host’s safety experts

Subcontracting to specialized engineers

### WP 6: Underground science

Deliverables

Physics potential of the infrastructure including a mix of three technologies

Design optimization (modularity, coverage)

More detailed simulations

Multi-disciplinarity and assessment of “other” sciences (biophysics, geophysics, geo-engineering)

Background studies

Milestones

Methods

Detailed detector response simulations (e.g. Monte-Carlo simulations of light propagation and detection)

### WP 7: Management and coordination

Link with USA, Japan

Outreach

# Relevance to the Objectives of the scheme

## Justification of the proposed Design Study

“l’union fait la force”

At the ApPEC “Munich meeting” held on November 2005, a coordinated effort among the 3 “liquids” has been proposed and accepted. Large detectors like Water Cherenkov,  Liquid Scintillator and Liquid Argon present important physics complementarities and also a lot of common R&D needs. They have to work in synergy.

The purpose is to develop conceptual design reports for European large scale liquid detectors into coherent and well-coordinated EU wide efforts towards a common physics goal and solving common problems together, taking into account the unique technological expertise in Europe and the other existing or planned programs in the world, such that mature designs and credible scenarios can be proposed around 2010.

Physics potential of the facility must be maximized

## Exploring the feasibility of the infrastructure

# Quality of the Management

## Management and Competence of the Participants

Table 3 Role of participants in tasks and research efforts and selected relevant references.

| Participant | References | Role in tasks and research effort |
| --- | --- | --- |
| IN2P3 | [1][2].. | WP1: WP2: …. |
| ETHZ | [3][4].. | WP1: WP2: …. |
| [1] J.E. Campagne et al., “xxx”, Phys. Rev. D.. [2] … [3] … |  |  |

## Justification of the Financing Requested

| WP1 | Underground infrastructure | 3.0 M€ |
| --- | --- | --- |
| WP2 | Underground tanks | 1.0 M€ |
| WP3 | Tank instrumentation | 2.5 M€ |
| WP4 | Pure liquid procurement | 1.0 M€ |
| WP5 | Safety and environment | 0.5 M€ |
| WP6 | Underground science and outreach | 1.5 M€ |
| WP7 | Management and coordination, Outreach | 1.0 M€ |
| Total |  | 10.5 M€ |

Table 4 Breakdown of the budget by participants and cost categories

| Participant | FTE | personnel | equipment | travel | other | sum |
| --- | --- | --- | --- | --- | --- | --- |
| IN2P3 |  |  |  |  |  |  |
| ETHZ |  |  |  |  |  |  |
| Sum |  |  |  |  |  | 3000 |