- Scientific background and objectives: The coverage of large areas of photodetection is a crucial element of experiments studying high energy atmospheric cosmic showers, neutrinos from different sources (Sun, atmospheric, supernova, reactor, accelerator) as well as the possible nucleon decay expected in Grand Unified Theory. The next generation of experiments can no longer be an enlargement of existing experiments which make use of large photmultipliers (PM) of large dimensions as well as a centralized acquisition system. One will need to rely on “distributed smart sensors” by analogy to the computing grid compared to supercomputers. The objective of this project is an “upstream research” for the realisation of large detectors using thousands of photomultipliers.  The partners propose to segment very large surfaces in macro pixels connected to an innovative electronics autonomous then allowing to reduce considerably the surface cost of these detectors and facilitating their industrialization. This development is made possible by the progress of microelectronics which allows integrating all the components for amplification and signal processing for dozens of channels in a chip (ASIC).  Only the relevant data will be sent out by network (eg. wireless) to the central data storage unit. It should be noted also that these developments, carried out here in collaboration with researchers of astroparticle physics have also other applications in the fields of the medical imaging and neutron imaging of dangerous nuclear matter.

- Description of project, methodology: The project will be guided by a progressive integration of the necessary functionalities.  A development of front-end electronics will be carried out starting from pre-existing elements in parallel of an improvement of the photomultipliers as well as the study of the transmission of the data by network.

- Expected results: The project will thus give results on the possibility of integrating in a modern technology (SiGe 0,35m) analogical and numeric electronics,  as well as on the cost of the detected photoelectron integrating the photomultiplier, its power supply, and the readout electronic. The main deliverable of the project is the construction of a "demonstrator" equipped with 16 PMs integrating the whole of the developed elements.

International and scientific context

There is a steady 25 years long tradition of water Cerenkov observatories having produced an incredibly rich harvest of seminal discoveries. The water Cerenkov movement was started in the early 80's by the scientists searching for proton decay. It fulfilled indeed this purpose by extending the proton decay lifetimes a few orders of magnitude.  Furthermore, water Cerenkov's, through a serendipitous turn, as frequently happens in physics, have also inaugurated:

- particle astrophysics through the detection of the neutrinos coming from the explosion of the Supernova 1987a  by IMB and Kamioka, acknowledged by the Nobel prize for Prof. Koshiba;

- the golden era of neutrino mass and oscillations by discovering hints for atmospheric neutrino oscillations while at the same time confirming earlier solar neutrino oscillation results.

The latest in the water Cerenkov series, the well known Super-Kamiokande, has now given strong evidence for a maximal oscillation between and , and several projects with accelerators have been designed to check this result. The results of the K2K experiment confirm the oscillation, and other experiments (MINOS in the USA, OPERA and ICARUS at Gran Sasso) should refine the oscillation parameters by 2010.

More recently, after the results from SNO and Kamland, a solid proof for solar neutrino flavour oscillations governed by the so-called LMA solution has been established. We can no longer escape the fact that neutrinos have indeed a mass, although the absolute scale is not yet known. Furthermore, the large mixing angles of the two above-mentioned oscillations and their relative frequencies open the possibility to test CP violation in the neutrino sector if the third mixing angle, , is not vanishingly small (we presently have only an upper limit at about 10° on , provided by the CHOOZ experiment). Such a violation could have far reaching consequences, since it is a crucial ingredient of leptogenesis, one of the presently preferred explanations for the matter dominance in our Universe.

The physicists have considered the possibility of producing new conventional neutrino beams of unprecedented intensity, made possible by recent progress on the conception of proton drivers with a factor 10 increase in power (4 MW with the CERN Super Proton Linac compared to the present 0.4 MW of the FNAL beam). These new neutrino ``super beams'' would explore down to 1 degree (i.e a factor 100 improvement on the  →e oscillation amplitude).

With the present design of the SPL, the energy of produced neutrinos is around 300 MeV, so that the ideal distance to study the   →e oscillations happens to be 130 km, that is exactly the distance between CERN and the existing Fréjus laboratory. The present laboratory cannot house a detector of the size needed to study neutrino oscillations, which is around 1 million cubic meters. But the recent decision to dig a second gallery, parallel to the present tunnel, offers a unique opportunity to realize the needed extension for a reasonable price.

Due to the schedule of the new gallery, a European project would be competitive only if the detector at Fréjus reaches a sensitivity on  around 1°, since other projects in Japan (T2K phase 1) and USA (NoVA) will have reached 2.5° by 2015. The working group has then decided to study directly a water Cerenkov detector with a mass approaching 1 megaton, necessary to reach the needed sensitivity. This detector has been nicknamed MEMPHYS (for MEgaton Mass PHYSics). Its study has benefited from a similar study by our American colleagues, the so-called UNO detector with a total mass of 660 kilotons. Simulations have shown that the sensitivity on  at a level of 1 degree could indeed be fulfilled with MEMPHYS.

Of course, MEMPHYS project is not the only one, since Japanese and American physicists have their own project, with similar potentialities. But owing to a new idea recently proposed by Piero Zucchelli, the European project could have unique characteristics which would make it very appealing. This idea is to send towards Fréjus, together with the SPL super beam, another kind of neutrino beam, called beta beam, made of e  or e produced by radioactive nuclei stored in an accumulation ring. If run simultaneously, super beam and beta beam, would allow studying the violation of CP symmetry in a much more efficient and redundant way than with other projects. This peculiarity, which would be European exclusivity, would give a considerable bonus to MEMPHYS concerning neutrino studies, since it could reach sensitivities on CP violation as good as those of a neutrino factory for    above 2 degrees.

As mentioned in the beginning, the megaton mass scale MEMPHYS detector will not only do the physics of neutrino oscillations, but would also address equally fundamental questions in particle physics and particle astrophysics.

In particular, such a detector could reach sensitivity around 1035 years on the proton lifetime, which is precisely the scale at which such decays are predicted by most supersymmetric or higher dimension grand unified theories, thus giving the hope for a fundamental discovery.

Such a detector would also bring a wealth of information on supernova explosions: it would detect more than 105 neutrino interactions within a few seconds if such an explosion occurs in our galaxy, and would observe a statistically significant signal for explosions at distances up to 1 Mpc, and provide a supernova trigger to other astroparticle detectors (gravitational antennas and neutrino telescopes). For galactic supernova explosions, the huge available statistics would give access to a detailed description of the collapse mechanism and neutrino oscillation parameters. In addition, the huge mass of the detector would allow detecting for the first time the relic neutrinos from past SN explosions.

The MEMPHYS detector based on water Cerenkov technics is indeed a multipurpose detector addressing several issues of utmost importance.

Objectives

The photodetection is a basic technique widely used in Astroparticules and Particles Physics. The next generation of experiments (MENPHYS as above mentioned, LENA and Km3 in Europe, UNO in the USA or HyperK in Japan) could not be any more the simple extrapolation of the present experiments (eg. SuperK, and Antares) for obvious reasons of cost and complexity.  The teams in the world are engaged in research and development programs in order to reduce the costs and complexity of such infrastructures.

Fig. 1: Assembly of 1m2 photodetectors

In this example, the basic module consists of sixteen photomultipliers of 8 inches.  Electronics treats locally the sixteen channels.

In this international competition, our upstream research will propose thanks to the spectacular developments of the micro-electronics and the communications to consider these new experiments as a network of distributed sensors.  We intend to develop new readout chip (ASIC) integrating from the pre-amplifier up to the digitization of the charge and time, as well as photomultipliers (PM) of medium dimensions (8 to 12 inches) produced in great series (MEMPHYS will need ~250,000 PMs), and the data network (eg. wireless). We want to instrument a demonstrator of 4x4 PMs constituting the macro-pixels of a true network (see Fig. 1).

The developments of ASICs for the readout of distributed photodetectors, fit in a more general research orientation of the "smart detectors" which associate sensor and readout electronics.  These systems require ASICs of increasingly complexity ("System one Chip") which associate many functionalities (amplification, digitalization, trigger...).  These developments are often common to other types of detectors (calorimeters, spectrometers, silicon detectors) what makes it possible to optimize the resources by irrigating the various fields.  In addition, the industrial applications of the "smart sensors" or PM with associated electronics ("light in, bits out") are developed within the framework of the GIS IN2P3-PHOTONIS.  Which confers on the project presented a broad visibility and many by-products in case of success.

B-2 – Description du projet et résultats attendus :

At the centre of the basic module presented in figure 1 which is a macro-detector (4x4 PMts) that will be the main deliverable of the project, it is shown the innovation that we propose to develop, that is to say an ASIC (Application Specific Integrated Circuit) with the following functionalities:

- High-speed discriminator to trigger on the single photoelectron

- Coincidence logic to strongly reduce the counting rate due to the noise

- Digitisation of the charge on 12 bits to provide numerical signals of the channels  which have triggered the system

- Digitisation of the time on 12 bits to provide temporal information with a precision of about the one nanosecond

- Variable gain to homogenize the photomultiplier responses and to thus use a common high voltage

Such an ASIC allows a strong cost reduction of the electronic necessary to the readout of large-sized detectors, as well as of the external components (high voltage, long cable of great quality...) since it is delivered numerical signals directly usable by the computers of data acquisition.  The data flow being strongly reduced locally could then be transmitted by wireless or transmission current.

Hold signal

Photomultiplicer

64 channels

Photons

Variable

Gain

Preamp.

Variable

Slow Shaper

20-100 ns

S&H

Bipolar

Fast Shaper

Unipolar

Fast Shaper

Gain correction

64*6bits

3 discriminator thresholds (3*12 bits)

Multiplexed

Analog charge output

LUCID

LUCID

S&H

3 DACs

12 bits

80 MHz encoder

64  Wilkinson 12 bit ADC

64 trigger

outputs

Multiplexed

Digital charge output

The LAL micro-electronic group had already acquired in the past a solid background in the design and the manufacture of ASICs in particle physics domains eg: the front-end and calibration electronics of the ATLAS  calorimeter for the LHC, the integrated readout  for the SiW calorimeter of the ILC...  In particular, one can underline the realization, in collaboration with the IPN of Lyon, of the readout ASIC of 64-channels Hamamatsu PMs of the target-tracker of the OPERA experiment (circuit OPERA_ROC with 32 channels, 13000 channels of photodétection).  This OPERA_ROC ASIC includes an auto-trigger at the level of a single photoelectron (pe) and the adjustment of the various channels of the multi-anodes PM as well as the charge linear readout up to 100pe. This ASIC was included in a more recent technology (BiCMOS SiGe 0.35 µm) in 64-channels (MOROC) for the luminometer of ATLAS using scintillating fibres.  New innovations are necessary and we identified the following objectives.

OBJECTIVES:

The present R&D proposal aims to continue the developments in three complementary fields:  digitalization of the signals, time measurements and introduction of a trigger logic.  The sensor readout network and the routing of the data must also be the subject of detailed studies while taking as a starting point the developments carried out by the IPN Lyon within the framework of the OPERA experiment. This study will take benefit of the LAPP team expertise.

The validation of the project implies necessarily measurements on a dedicated prototype under real conditions of detection.  This can be done by a water tank for one (or two) detection module and the external system to trigger on cosmic rays.  The long expertise of the IPNO team on such a task will benefit to the project. One of the last IPNO implications in this field is the developments done for the AUGER experiment. The UHE cosmic shower surface detectors composed on tanks filed with water and  instrumented with 3 PMs each developed by PHOTONIS for which IPNO had studied a dedicated PMT base including a very low  power high voltage module. The production (5000 pieces) is now completed and has been shared between IPNO and INFN Torino.

Last but not least, such a project requires from the very beginning to take into account the industrialization of the photodetectors and the associated elements (electronics, high voltage…) to produce very great surfaces at a reasonable cost, this is why the expertise of the PHOTONIS Company is essential as well as the long past relationship between IN2P3 laboratories and this French Company one of the leader in this sector (the competitor is the Japanese Hamamatsu Company for sure).

The great difficulty in associating very fast analogical electronic and the digitalization on a broad dynamic range did not make possible yet to integrate all these functions in only one single integrated circuit, it is thus a challenge which we propose to raise.  We will base our project on certain parts which were already developed separately and on the evolution of integrated technologies, in particular BiCMOS SiGe 0.35µm now allowing to consider such circuits.

The project will be carried out in 3 phases:

- Study and realization of a prototype of readout ASIC

An extension of ASICs MOROC realized for the PM multi-anodes of the luminometer of the ATLAS experiment should make it possible to work out a prototype usable on a matrix of "macro-sensors".  This ASIC would be carried out for the end 2006 and available for the tests in 2007.  In parallel, the studies on the sensor network (eg. Wireless) would be undertaken.  This study would be taken in charge by the LAPP.  However, taking into account the charge of the laboratory, the implication in 2006 will be devoted to conceptual studies, the achievements will be done in 2007.

- Study and realization of a prototype of large-sized photodetector

After realization and characterization of the readout ASIC, several measurements with a prototype equipped with representative of final PMs are considered.  This prototype of full-scale tests requires work on tight coating (potting), the polarization of the high voltage and the implementation of electronics.  One can consider PMs prepared by PHOTONIS Company within the framework of the works in progress with the IN2P3. This prototype of macro-sensors would be studied and constructed by the IPN Orsay in 2007. It would be tested in a box reflecting the experimental conditions.

- “final" ASIC realisation

Thanks to results obtained with the above mentioned prototypes, a set of few samples of a version that can be manufactured would be produced for validation at the end of 2007.