DATA CALIBRATION: DAB

JEC-AST, Nov 2011

LAL, Orsay

The DAB (“diode a bruit”) is used for calibration in the NRT, i.e. determining the value of the signals measured in Janskys.

We investigate the utilization of the DAB to calibrate the data. We show that the calibration introduces errors in the data obtained.

We also study the frequency dependence of the DAB signal.

DAB calibration

This is clearly seen in the Abell 1205 data. We show the ON-OFF signal in the 21-cm line vs. data cycle in the following cases: non calibrated, and calibrated using calibration coefficients per run and calibrations coefficients per cycle. The calibration coefficients per run are the mean of the ones obtained in each cycle.

We see that the non-calibrated data are the most stable. The calibration coefficients per run are different from each other, which translates into a signal evolution showing a pattern which depends on runs. The calibration per cycle does not introduce any pattern in the signal, but in some cycles it yields a signal value quite different from the bulk. The cycles where this is seen are different in each channel.

In the case of Abell 2440 we find:

The results are similar to that obtained for Abell 1205. The non-calibrated data are the most stable. The calibration per run shows no clear dependence on the run but for the first (15th April 2011) and last (13th June 2011) runs. The calibration per cycle shows ‘bad’ cycles which are different in both channels.

We could take two different approaches to solve the instability of the DAB calibration:

- Use one mean calibration coefficient per channel for the whole dataset. However, this would introduce a bigger error in the Jansky estimation (as it will include all the coefficient variations occurred during the whole observation period).

- Keep the calibration per cycle. In this case, one could just dismiss the ‘bad’ cycles from the analysis or set the coefficients in the ‘bad’ cycles equal to the mean of the adjacent cycles, which is a bit of ‘cooking’.

DAB Chromaticity

The DAB shows a chromatic response, which is independent of the electronic chain used to acquire the data.

We show below the DAB spectrum obtained for the “Voie E” during the observation of the PKS1127-14 radio source, using WIBAR (ROACH card) in the frequency band [1100,1580] MHz. The resulting spectrum has been normalized by the system-passing band.

In the following plot we include the DAB spectrum obtained from the observations of Abell 1205 using the BAO electronics, for both channels Ch0 (blue) and Ch1 (red).

The curve for each channels is the mean of the two DAB steps. To superimpose these curves to the one in Janskys, we have multiplied arbitrarily Ch0 by a factor 18 and Ch1 by a factor 23.

The resemblance of the spectra is quite remarkable, given that in the first case the analogic signals are transmitted through cables to the ADC, whereas the BAO electronics makes the digitization in the chariot.