MEMO

Authors: J.E Campagne. A.S. Torrentó

Nançay/Amas/16.03.12

Diffusion : R. Ansari, P. Colom, D. Charlet, E. Gérard, Ch. Magneville, J. M. Martin, M. Moniez

Objet: Electronic noise in the BAO spectra taken during the period Apr. 2011-Jan. 2012

# Introduction

The oscillations studied in reference Nançay/Amas/010.02.12 are clearly limiting the sensitivity of the data taking, here we report on the presence of several RFI noise lines in the BAO spectra obtained from the observations of all clusters during the period April 2011 up to January 2012 (for definiteness we focus on Abell 1205 data). The data processing software includes an RFI-cleaning procedure (median filtering) which has proven to be very effective for transient perturbations but still leaves quasi-permanent RFI traces in the spectra, especially in the case of noise generated by electronic devices. It is clear that we can cut in the spectra some frequency windows but we think that these RFIs are the signal of non-understood electronic phenomena.

Electronic RFI lines are characterized by a very short bandwidth (~100kHz) and no tails, as opposite to information-carrier signals which are wider (>1MHz) and show some structure around (for example, radars, TV/radio broadcast and portable phones).

In this note we discuss the noise found in our data, namely at 1250+k*25 MHz and at other a priori isolated frequencies.

# RFI at 1250+k* 25 MHz

We have clearly identified in our data electronic RFI lines at frequencies 1250+k*25 MHz. We show in Figure 1 an OFF/OFFfilt spectrum () averaged over 1216 cycles from Abell 1205, measured by Ch1. The power (vertical axis) is presented in dB units (ie. defined as 10*Log10(P) = 20* Log10(Volt)).

Figure 1. 1250+k*25 MHz RFI lines in an OFF/OFFfilt spectrum averaged over 1216 cycles from Abell 1205, Ch1. The vertical axis is in dB defined as 10*Log10(P).

In the band [1250,1500] MHz we find 9 lines at 1275, 1300, 1325, 1350, 1375, 1400, 1425, 1450, 1475 MHz, although the lines at 1300, 1350, 1400, 1450 are much stronger (we cannot measure a reliable value on the limits of the band, i.e. 1250 and 1500 MHz). We study the time evolution of these lines, plotting the integrated power in the band [line-1,line+1] MHz vs. cycle number for both channels (see Figure 6).

Figure 2. Time evolution of the 1250+k*25 MHz lines in Ch0 (top) and Ch1 (bottom).

We observe that these RFIs are important until cycle ~380 (16th Jun 2011). The next data, taken on 7th Jul 2011, are much less affected: Ch0 still shows some power at 1375, 1400 and 1450 MHz whereas in Ch1 the lines disappear almost completely. It should be noted the different behavior of the RFI at 1275 MHz, with big spikes at some specific points in time. The mess observed at ~700 cycles is the already mentioned “bad period” from 22nd Aug 2011 to 15th Sep 2011 (ref. Nançay/Abell1205/23.11.11).

We know (mail communication from C. Pailler) that on 4th Jul 2011, 3 servers for FAN (bao3, bao5 and bao6) had been installed in Nançay. However, we do not know if other changes concerning the “Raw” cards or the cables were done that day. We recall that we had also remarked a change in the gain pattern at the beginning of July 2011 (Figure 3).

Figure 3. Changes in the gain pattern at the beginning of July 2011.

We show in Figure 4 and Figure 5 the time evolution of the lines already shown in Figure 2, but in separate plots for Ch0 (left) and Ch1 (right) to see more in detail.

Figure 4. Time evolution of RFI lines at (from top to bottom): 1275, 1300, 1325, 1350 and 1375 MHz, for Ch0 (left) and Ch1 (right).

Figure 5. Time evolution of RFI lines at (from top to bottom): 1400, 1425, 1450 and 1475 MHz, for Ch0 (left) and Ch1 (right).

From these plots, we can extract the following information:

- Besides the step at ~400 cycles and the bad period at ~700 cycles, in general the power of the RFIs in both channels seems not to be correlated, but...

- Sometimes, the signal disappears from one channel to appear in the other one. This happens, for example, at 1375 MHz (cycle ~950, 16th Nov 2011) and 1400 MHz (cycle ~380, 7th Jul 2011).

- The changes in power are sharp, which correspond to a change in the noise conditions from run to run.

The origin of these lines is still unknown. The first culprit we could think of is the DISTCLOCK module, which provides a 50-MHz clock signal to the acquisition system. However, in the bench test at LAL we have been able to excite a set of RFI lines at =50 MHz when a 1350-MHz signal is directly injected to the ADC Raw card. We do find a 1350-MHz signal in our cluster data which corresponds to the radar of the Roissy aeroport. This signal could also have generated the noise observed.

# RFI at other frequencies

## 1406.2 MHz

We show in Figure 6 the [1390, 1410] MHz frequency band in ON/OFFfilt (Ch0 cyan, Ch1 orange) and OFF/OFFfilt (Ch0 blue, Ch1 red) spectra averaged over 1216 cycles for Abell 1205. In this band we can see the electronic RFI at 1400 MHz (thin and clean shape) and a multi-lines system centered at 1406.2-MHz with a substructure (peaks with ~200 kHz).

Figure 6. RFI lines at 1400 and 1406 MHz in ON/OFFfilt (Ch0 cyan, Ch1 orange) and OFF/OFFfilt (Ch0 blue, Ch1 red) spectra averaged over 1216 cycles for Abell1205.

In Figure 7 we show the evolution of the mean power in the [1405,1408] MHz band. We notice that this RFI is constantly present in Ch1, whereas it is nearly absent in Ch0. Contrary to most of the noise signals shown in the previous section, this line seems to be re-activated at the beginning of July 2011. The 17th Oct 2011, this RFI disappears from Ch1 and is detected in Ch0 with the same power it had in Ch1. The rest of the time, Ch0 is not affected by this RFI. The increase in power just before the bad period, is concurrent with the solar noon happening <1h after the beginning of the observations.

Figure 7. Time evolution of the mean power in the [1405, 1408] MHz band from ON/OFFfilt (Ch0 blue, Ch1 red) spectra averaged over 1216 cycles for Abell1205.

The origin of this signal is also unknown. One possibility is that it might have been picked up after the analog module, just before the ADC. In this case the noise frequency would have been added up to the 1250 MHz of the local oscillator, making out the 1406 MHz. Hence the coupled noise would have a frequency of approximately 1406 - 1250 = 156 MHz.

As the line is present all the time and its shape indicates that it could be an information-carrier signal, we have searched for signal candidates with this frequency. We have found that the maritime services have allocated the frequencies between 156 and 161 MHz. In particular, the frequency 156.2 MHz is used for the CROSS safety and calling frequency (see for instance  http://www.anfr.fr/fileadmin/mediatheque/documents/radiomaritime/Affectation_VoiesVHF.pdf).

# Discussion

The fact that the intervention in Nançay has probably produced major changes in the noise conditions, we have listed below some key dates for the system to see if we find any correlation between the effects seen in the data and any external event:

| Date | Effect |
| --- | --- |
| 7th Jul 2011 | Change in gain Dramatic decrease of 25-MHz RFI |
| 17th Oct 2011 | Disappearance of 5-MHz oscillation “Jump” of RFI(1406-MHz) from Ch1 to Ch0 |
| 7th Aug – 5th Sep 2011 | Power increase in all the spectrum (solar noon) |
| 22ndAug - 15th Sep 2011 | Bad period |
| 16th Nov 2011 | Re-activation/disappearance of some 25-MHz lines in Ch0 Re-activation of all 25-MHz lines in Ch1 (at a level lower than before 7th Jul 2011) |

In particular, it is remarkable to see the disappearance of the 5-MHz oscillation pattern in the middle of October 2011, as shown in Figure 8, which still is under investigation.

Figure 8. On/Gain spectra for Abell 1205, cycle no. 10, Ch0, taken on 5/10/2011 (top) and 17/10/2011 (bottom). The top plot is representative of the conditions before this date, in the same way the bottom plot is representative of the conditions after.

# Conclusions

The presence of RFI lines in the spectra add difficulties to the detection of the HI signal from galaxy clusters. The lines can have been generated by electronic devices or by external systems such as radars, radio emitters, TV/radio broadcast, etc.

The RFI-cleaning algorithm developed for the data processing so far does not completely remove this noise, especially if it is quasi-permanent. That is to say if this noire represents more than half of the paquets in the median filter time window (5120 paquest = 0.6sec). So we are trying to identify the sources to see if we could suppress the noise emission: electronics, human emissions…

We have identified in our data, sharp RFI lines at 1250+k*25 MHz and a wider line at 1406 MHz.

The time evolution of the integrated power of both RFIs shows that a major change has occurred in the acquisition system at the beginning of July 2011, which is concurrent with human intervention in Nançay. Thus we have listed some key dates for the system to try to correlate external events with the changes in noise conditions observed in the data.

The origin of these RFI is unknown. In the case of the 1250+k*25 MHz lines we have identified two possible culprits: the DISTCLOCK module which provides a 50-MHz clock to the acquisition or the 1350-MHz signal from Roissy aeroport which can excite these modes in some electronic component of the ADC card. The 1406-MHz RFI might be produced by a 156-MHz signal picked-up after the mixer. We have found that this frequency is used for maritime distress and communications. Of course other explanations may be found more relevant…

OFFfilt means the usual OFF spectra median filtered with a 2.1MHz window